SIFF 2026

Singapore International Food Forum 2026

Advanced Thermal Technologies for Sustainable Rice Parboiling: Impact of Ohmic Heating, Microwave, and Ultrasound Treatments on Rice Quality

Dr. K.A. Athmaselvi

Professor, Centre of Excellence for Grain Sciences, Department of Food Process Engineering, National Institute of Food Technology, Entrepreneurship and Management, Thanjavur (NIFTEM-T). Thanjavur – 613005. India.

Email: athmaselvi@iifpt.edu.in

Abstract

Rice parboiling is widely employed to improve milling efficiency, cooking quality, and storage stability; however, conventional methods are often associated with high energy consumption, prolonged processing time, and uneven heat transfer. This study evaluates the effectiveness of advanced parboiling technologies, namely Ohmic Heating (OH), Microwave Heating (MW), and Ultrasound-assisted parboiling (US), as sustainable alternatives to conventional steaming for rice processing. The influence of these technologies on hydration kinetics, milling characteristics, physicochemical properties, cooking quality, starch structural modifications, volatile compounds, and storage stability was investigated. Advanced treatments significantly enhanced processing efficiency by reducing soaking and heating duration while improving overall rice quality. Among the evaluated methods, ultrasound-assisted parboiling achieved the highest head rice yield (99.40%), indicating superior milling performance. Ohmic heating exhibited improved grain hardness, reduced chalkiness, enhanced shelling efficiency, and better nutrient retention due to rapid and uniform volumetric heating [1]. Microwave treatment considerably reduced processing time and contributed to desirable cooking characteristics [2]. Structural analyses using SEM, FTIR, XRD, and DSC confirmed starch gelatinization, recrystallization, and modifications in crystalline structure induced by advanced thermal treatments. Ohmic heating showed pronounced starch restructuring and improved cellular integrity, while ultrasound treatment influenced amylose behavior through cavitation effects [3]. Furthermore, the advanced technologies reduced energy consumption and improved storage stability compared with conventional processing. Overall, the findings demonstrate that OH, MW, and US technologies provide promising, energy-efficient, and environmentally sustainable solutions for modern rice parboiling with strong potential for industrial-scale commercialization.

References

[1] Kaur, N., & Singh, A. K. (2016). Ohmic Heating: Concept and Applications—A Review. Critical Reviews in Food Science and Nutrition, 56(14), 2338–2351. https://doi.org/10.1080/10408398.2013.835303

[2] Rockembach, C. T., de Oliveira, M., et al. (2019). Microwave Parboiling: Reduction in Process Time, Browning of Rice and Residual Phosphorus Content in the Waste Water. Journal of Food Science, 84(8), 2222–2227. https://doi.org/10.1111/1750-3841.14738

[3] Yang, W., Kong, X., Zheng, Y., et al. (2019). Controlled Ultrasound Treatments Modify the Morphology and Physical Properties of Rice Starch Rather than the Fine Structure. Ultrasonics Sonochemistry, 59, 104709. https://doi.org/10.1016/j.ultsonch.2019.104709