SIFF 2026

Singapore International Food Forum 2026

Oxy Thru 0.2L as a Reliable Vessel for
Multi-Species Cultivated Meat Production

GA Helenaa, Y. Takagakib, K. Shiotsuka a, I. Shin a, K. Hirano a , N. Funamotob,  S. Tatsumib,  M. Yui a, I. Kawashima a , Y. Hanyu a

a Integriculture Inc., Shonan Health Innovation Park A32F 3111, 2-26-1 Muraokahigashi, Fujisawa, Kanagawa 251-8555, Japan

b Sumitomo Riko Co., Ltd., JP Tower Nagoya, 1-1-1 Meieki, Nakamura-ku, Nagoya-shi, Aichi 450-6316, Japan

Abstract

Advancements in cell-cultured food are currently hindered by the lack of specialized hardware to maintain consistent, convenient, and high-quality yields [1, 2]. There is an urgent need for reliable, specialized culture vessels that bridge the gap between bench-top research and industrial scaling without compromising biological output. [3]. The Oxy Thru 0.2 L cultivator addresses this challenge as a solution designed to meet the diverse metabolic demands of cellular agriculture across various species. The system pairs a gas-permeable PDMS (Polydimethylsiloxane) vessel with a packed-bed culture system that utilizes edible scaffolds. This combination aims to provide robust gas exchange facilitation and increased surface area for adherent cell growth [4]. Preliminary research demonstrates that the static culture Oxy Thru 0.2 L offers superior performance and versatility relative to traditional static 100 mL polystyrene reactor. Using this packed bed, scaffold-based culture approach, we evaluated 3 types of cell lines, avian (DF1), ungulate (MDBK), and teleost (FHM CCL-42) cell lines. The results showed that all tested cell types were able to proliferate within the vessel, yielding both significantly viable and total cell number/gram scaffold than those in static polystyrene reactor. These results demonstrate that the Oxy Thru 0.2 L provides the environment necessary to scale diverse cell types. By overcoming critical reliability constraints in cellular agriculture, this system may provide a scalable framework for more efficient cultivated meat production.

References

[1] C. J. Ngwa, K.-H. Chiu, K. J. Brenner, C. Rodrigues, Z. Peršin Fratnik, J. Vajda, B. Vihar, J. Morales-Dalmau, F. Castelo Ferreira, M. Henkel, and S. Schillberg, “Bioreactor parameters and systems for cultured meat production,” Future Foods, Vol. 12, p. 100796, 2025.

[2] M. Li, L.-Y. Chin, S. Shukor, A. Tamayo, M. V. Maus, and B. Parekkadan, “Closed loop bioreactor system for the ex vivo expansion of human T-cells,” Cytotherapy, Vol. 22, No. 5, pp. 273-281, 2020. 

[3] P. Bajgain, R. Mucharla, J. Dindzans-Kulkarni, L. Guo, and J. F. Vera, “Optimizing the production of suspension cells using the G-Rex ‘M’ series,” Molecular Therapy: Methods & Clinical Development, Vol. 1, p. 14015, 2014.

[4] C. J. Ochs, J. Kasuya, A. Pavesi, and R. D. Kamm, “Oxygen levels in thermoplastic microfluidic devices during cell culture,” Lab on a Chip, Vol. 14, No. 3, pp. 459–462, 2014.

* Corresponding author E-mail address: grace.helena@integriculture.com